PR 5 Resistance of a resistance wire and its length

Suggested Markscheme (Total = 15 marks)

Record of measurements and calculations 		                                   	[3]

- Headings with units with at least 6 sets of readings for l , I, V, R

- Acceptable values and correct d.p for l, I, V

- correctly calculated and correct s.f. for R

- Wide range (at least 70 cm) and fairly well spread values of length l

Record of diameter

- Acceptable values and correct d.p for d1, d2, d3	: 0.39 mm = 0.039 cm	[1]

- Accurate calculation and correct d.p. for <d>	(2 d.p.)          (3 d.p.)	[1]

Graph	 (printed and attached to this practical):  correct labelling of axes		[4]

Gradient  G 	= within 20% of teachers' value

= 0.10  0.02  (/cm) or 	10  2  (/m)					[1]

Resistivity

- Accurate substitution of values into formula						[1]

- = 1.2 x 10-6 	 m or 	1.2 x 10-4 	 cm					[1]

Conclusion

R is directly proportional to l.			[start graph from origin!]		[1]

(straight line passes through/close to origin)

Precautions taken (any one)						 		[1]

- The circuit was closed only when readings were taken. This prevented the resistance wire from heating up, which could raise its resistance and cause inaccurate results.

- All connections were tightened

  - to prevent the readings from fluctuating due to poor connections OR

  - so that the total resistance of the circuit was not affected (hence affecting the current).

- It was checked that the resistance wire was mounted tautly across the ruler so that the length l used corresponds exactly to the measured valued.

- The jockey was held perpendicularly to make a light contact with the resistance wire, to avoid changing its cross-sectional area (and hence its resistance).

Possible source of error (any one)							[1]

- The internal resistance of the plug switch / ammeter / accumulator / connecting wires is not negligible (any one object) Hence, the total resistance of the circuit was affected.

- The internal resistance of the voltmeter was not infinite. A small current could flow through the voltmeter. Hence the current measured by the ammeter was not the current flowing through the resistance wire.

- The resistance wire had kinks (bends) along its length.

  - Its effective length measured is not accurate OR

  - Its cross-sectional area is not uniform, affecting its resistance.